Lab 1：Introduction

| Author | Name： 崔俞崧 赵奕帆 Student ID: 11811305 11812011 |
| --- | --- |
| Introduction - In the problem 2.4, we verified the commutative, associative and distributive properties of convolution for a specific set of signals. And mainly uses the conv() to solve this problem. - In the problem 2.5, we mainly use the conv() and filter() functions to solve the problems. We learned the system properties of linearity and time-invariance. - In the problem 2.10, we learned how to remove the echo of a recording of speech signal. Lab results & Analysis： Problem 2.4 The figure is shown below The output results are as below. We can figure out that the answer is regardless of the order. The two methods gave the same result. So the distributive property is verified. The results are the same. So the associative property is verified. As shown below, we can see that when using the commutative property, the outputs are the same if changing the input and impulse response of the system ye1n and ye2[n]. And we can know that ye1n-n0= ye2[n]. We can see from the figure below that yf1[n]≠yf2[n]. But it couldn’t prove that the associative property is wrong. The reason is that System 1: y[n] = (n+1)x[n] is not a LTI system. So we can’t apply the associative property. We can see from the figure below that yg1 is not equal to yg2. But it can’t prove the distributive property of convolution is wrong. The reason is that System 1: yn=x2[n] is not a LTI system. So we can’t apply the distributive property. 2.5 (a). (b). As shown in the Figure 2.5(a) and Figure 2.5(c), System 1 and System 3 is linear. w3[n] and w4[n] have the same output. w3n=w4n=w1n+2*w2n, and x3n=δn+2δn-1=x1n+2x2n, so System 1 is linear. z3n=z4n=z1n+2*z2[n], and x3n=δn+2δn-1=x1n+2x2n, so System 3 is linear. As shown in the Figure 2.5(b), System 2 is not linear. Since y3[n] and y4n don’t have same output. y3n=cos⁡(x1n+2x2n)≠y4n=y1n+2*y2n, so System 2 is not linear. (c). As shown in the Figure 2.5(a) and Figure 2.5(b), System 1 is time-invariant. We can see the graphs of w1[n], w2n and the graphs of y1[n] and y2n. The graph of w2n is same to the graph of w1n after translation. The graph of y2n is same to the graph of y1n after translation. As shown in the Figure 2.5(c), System 3 isn’t time-invariant. The graph of z2n isn’t same to the graph of z1n after translation. z1n=nx1[n], z2n=nx2n=nx1n-1, but zn=(n-1)x1n-1. (d). System 1: A = [1 -3/5]; B = 1; x = [1 zeros(1,19)]; h1 = filter(B, A, x); System 2: for i = 2:21 , h2(i) = ((3/5)^(i-2)) * h2(i-1) + x(i-1) ,end; h2 = h2(2:end); (e). System 1: A = [1 -3/5]; B = 1; u = [ ones(1,20)]; s1 = filter(B, A, u); System 2: for i = 2:21, s2(i) = ((3/5)^(i-2)) * s2(i-1) + u(i-1), end; s2 = s2(2:end); (g). s1n=z1[n]. Because the System 1 is a LTI system. So, for not LTI system: System 2, s2n=z2[n]. 2.10 As shown in this figure, he = δn+0.5δ[n-1000]. As shown in the figure, z[n] = x[n] is a valid solution. The reason is that y[n] z[n]*he[n]. As shown in the figure, it is her. This figure is the sound which cannot hear the echo. The result is not a unit impulse. The echo removal system should have infinite-length impulse response, but our system is not infinite. For y: As shown in the figure, we can find that for y, N=1000 and we can calculate Ryy[n]: Ryyn=xn*δn+αδn-N*x-n*δn+αδn+N =Rxx[n]*((1+α2)δ[n]+αδ[n-N]+αδ[n+N]) So, we can get that: Ryy0=Rxx0*1+α2δ0+αδ-1000+αδ1000 =1+α2Rxx0+αRxx-1000+αRxx1000 =(1+α2)Rxx0+2αRxx1000 So, α2Rxx0+2αRxx1000+(Rxx0-Ryy0)=0 is an equation, and we could get α using the extract roots formula. So the answer is α=0.5 For y2: As shown in the figure, for y2, N=501 and we could calculate the Ryy[n]: Ryyn=xn*δn+αδn-N*x-n*δn+αδn+N =Rxx[n]*((1+α2)δ[n]+αδ[n-N]+αδ[n+N]) So, Ryy0=Rxx[0]*((1+α2)δ[0]+αδ[-501]+αδ[501]) =1+α2Rxx0+αRxx-501+αRxx501 =(1+α2)Rxx[0]+2αRxx501 So, α2Rxx0+2αRxx501+(Rxx0-Ryy0)=0 is a binary first order equation, we can get α using extract roots formula. So the answer is α=0.7 As shown in the figure, we can find for y3, N1=751, N2=2252 and calculate Ryy[n]: Ryy[n]=x[n]*(δ[n]+α1δ[n-N1]+α2δ[n-N2])*x[-n]*(δ[n]+α1δ[n+N1]+α2δ[n+N2]) =Rxxn*1+α12+α22δn+α1δn+N1+α1δn-N1+α2δn+N2+α2δn-N2+α1α2δn-N1+N2+α1α2δn-N2+N1 Ryy[0]=(1+α12+α22)Rxx[0]+2α1Rxx[751]+2α2Rxx[2252]+2α1α2Rxx[1501] And RyyN1, RyyN2. Then we can get α1=0.75, α2=0.6. Finally, we use sound (y, 8192), sound (y2, 8192), sound (y3, 8192) could find out that there is no echo. Codes: 2.4 % Problem 2.4(a) x1 = [1 1 1 1 0 0 0 0 0 0]; nx1 = 0:1:9; h1 = [1 -1 3 0 1]; h2 = [0 2 5 4 -1]; nh1 = 0:1:4; subplot(3, 1, 1), stem(nx1, x1), title("x_1[n]"); subplot(3, 1, 2), stem(nh1, h1), title("h_1[n]"); subplot(3, 1, 3), stem(nh1, h2), title("h_2[n]"); saveas(gcf, "plots/P2_4_a.png"); close; % Problem 2.4(b) x1 = [1 1 1 1 0 0 0 0 0 0]; nx1 = 0:1:9; h1 = [1 -1 3 0 1]; nh1 = 0:1:4; y1 = conv(x1, h1); y2 = conv(h1, x1); ny = nh1(1)+nx1(1):1:nh1(end)+nx1(end); subplot(2,1,1), stem(ny, y1), title("y1[n]=x1[n]*h1[n]"); subplot(2,1,2), stem(ny, y2), title("y2[n]=h1[n]*x1[n]"); saveas(gcf, "plots/P2_4_b.png"); close; % Problem 2.4(c) x1 = [1 1 1 1 0 0 0 0 0 0]; nx1 = 0:1:9; h1 = [1 -1 3 0 1]; h2 = [0 2 5 4 -1]; nh1 = 0:1:4; y11 = conv(x1,h1); y12 = conv(x1,h2); y1 = y11+y12; h12 = h1+h2; y2 = conv(x1,h12); ny = nh1(1)+nx1(1):1:nh1(end)+nx1(end); subplot(2, 1, 1), stem(ny, y1), title("y1 = (x[n]*h1[n]+x[n]*h2[n])"); subplot(2, 1, 2), stem(ny, y2), title("y2 = x[n]*(h1[n]+h2[n])"); saveas(gcf, "plots/P2_4_c.png"); close; % Problem 2.4(d) x1 = [1 1 1 1 0 0 0 0 0 0]; nx1 = 0:1:9; h1 = [1 -1 3 0 1]; h2 = [0 2 5 4 -1]; nh1 = 0:1:4; y1 = conv(conv(x1, h1), h2); y2 = conv(x1, conv(h1, h2)); ny = nh1(1)+nx1(1):1:nh1(end)+nh1(end)+nx1(end) subplot(2,1,1), stem(ny, y1), title("yd1[n]=(x_1[n]*h_1[n])*h_2[n]"); subplot(2,1,2), stem(ny, y2), title("yd2[n]=x_1[n]*(h_1[n]*h_2[n])"); saveas(gcf, "plots/P2_4_d_out.png"); close; % Problem 2.4(e) x1 = [1 1 1 1 0 0 0 0 0 0]; nx1 = 0:1:9; h1 = [1 -1 3 0 1]; h2 = [0 2 5 4 -1]; nh1 = 0:1:4; he1 = h1; he2 = [0 0 h1]; nhe2 = 0:1:6; nye1 = nh1(1)+nx1(1):1:nh1(end)+nx1(end); nye2 = nhe2(1)+nx1(1):1:nhe2(end)+nx1(end); ye1 = conv(x1, he1); ye1_inter = conv(he1, x1); ye2 = conv(x1, he2); ye2_inter = conv(he2, x1); subplot(3,1,1); hold on stem(nye1, ye1, 's'); stem(nye1, ye1_inter, 'p'); legend('y_{e1}[n] = x_1[n]*h_{e1}[n]', 'y_{e1}[n] = h_{e1}[n]*x_1[n]', 'Location', 'northeast'); subplot(3,1,2); hold on stem(nye2, ye2, 's'); stem(nye2, ye2_inter, 'p'); legend('y_{e2}[n] = x_2[n]*h_{e2}[n]', 'y_{e2}[n] = h_{e2}[n]*x_2[n]', 'Location', 'northeast'); subplot(3,1,3); hold on stem(nye1+2, ye1, 's'); stem(nye2, ye2, 'p'); legend('y_{e1}[n-n_0]', 'y_{e2}[n]', 'Location', 'northeast'); saveas(gcf, "plots/P2_4_e.png"); close; % Problem 2.4(f) x1 = [1 1 1 1 0 0 0 0 0 0]; nx1 = 0:9; h1 = [1 -1 3 0 1]; h2 = [0 2 5 4 -1]; nh1 = 0:4; w = x1.*(nx1+1); hw = nx1; yf1 = conv(w, h1); nyf1 = (hw(1)+nh1(1)):1:(hw(end)+nh1(end)); hf1 = [1 0 0 0 0].*[1 2 3 4 5]; nhf1 = 0:1:4; hf2 = h1; nhf2 = nh1; hseries = conv(hf1, hf2); nhseries = (nhf1(1)+nhf2(1)):(nhf1(end)+nhf2(end)); yf2 = conv(x1, hseries); nyf2 = (nx1(1)+nhseries(1)):(nx1(end)+nhseries(end)); figure; hold on stem(nyf1, yf1, 's'); stem(nyf2, yf2, 'p'); title('y_{f1}[n] & y_{f2}[n]'); legend('y_{f1}[n]', 'y_{f2}[n]', 'Location', 'northeast'); xlabel('n'); saveas(gcf, "plots/P2_4_f.png"); close; % Problem 2.4(g) x1 = [1 1 1 1 0 0 0 0 0 0]; nx1 = 0:1:9; h1 = [1 -1 3 0 1]; h2 = [0 2 5 4 -1]; nh1 = 0:1:4; nh2 = nh1; xg = [2 0 0 0 0]; nxg = 0:1:4; yga = xg.^2; nyga = nxg; hg2 = h2; nhg2 = nh2; ygb = conv(xg, hg2); nygb = (nxg(1)+nhg2(1)):1:(nxg(end)+nhg2(end)); yg1 = [yga 0 0 0 0] + ygb; nyg1 = nygb; hg1 = [1 0 0 0 0].^2; nhg1 = nxg; hparallel = hg1 + hg2; yg2 = conv(xg, hparallel); nyg2 = nyg1; figure; hold on; stem(nyg1, yg1, 's'); stem(nyg2, yg2, 'p'); title('y_{g1}[n] & y_{g2}[n]'); legend('y_{g1}[n]', 'y_{g2}[n]', 'Location', 'northeast'); saveas(gcf, "plots/P2_4_g.png"); close; 2.5 2.5(a) x1 = [1 0 0 0 0 0]; x2 = [0 1 0 0 0 0]; x3 = [1 2 0 0 0 0]; nx = 0:5; xe1 = [0 0 x1]; xe2 = [0 0 x2]; xe3 = [0 0 x3]; w1 = x1; w2 = x2; w3 = x3; for i = 3:8 w1(i-2) = xe1(i) - xe1(i-1) - xe1(i-2); w2(i-2) = xe2(i) - xe2(i-1) - xe2(i-2); w3(i-2) = xe3(i) - xe3(i-1) - xe3(i-2); end w4 = w1 + 2*w2; subplot(3, 1, 1), stem(nx, w1), set(get(gca, 'Title'), 'String', 'w_1[n]'); subplot(3, 1, 2), stem(nx, w2), set(get(gca, 'Title'), 'String', 'w_2[n]'); subplot(3, 1, 3) hold on stem(nx, w3, 'b--s') stem(nx, w4, 'r--p') set(get(gca, 'Title'), 'String', 'w_3[n] and w_4[n]'); legend('w_3[n]', 'w_4[n]', 'Location', 'southeast'); saveas(gcf, "P2_5_a_1.png"); 2.5(b) x1 = [1 0 0 0 0 0]; x2 = [0 1 0 0 0 0]; x3 = [1 2 0 0 0 0]; nx = 0:5; w1 = cos(x1); w2 = cos(x2); w3 = cos(x3); w4 = w1 + 2 * w2; subplot(3, 1, 1), stem(nx, w1), set(get(gca, 'Title'), 'String', 'w_1[n]'); subplot(3, 1, 2), stem(nx, w2), set(get(gca, 'Title'), 'String', 'w_2[n]'); subplot(3, 1, 3) stem(nx, w3) hold on stem(nx, w4, 'r') set(get(gca, 'Title'), 'String', 'w_3[n] and w_4[n]'); legend('w_3[n]', 'w_4[n]', 'Location', 'southeast'); saveas(gcf, "P2_5_a_2.png"); 2.5(c) x1 = [1 0 0 0 0 0]; x2 = [0 1 0 0 0 0]; x3 = [1 2 0 0 0 0]; nx = 0:5; xe1 = [0 0 x1]; xe2 = [0 0 x2]; xe3 = [0 0 x3]; w1 = x1.*nx; w2 = x2.*nx; w3 = x3.*nx; w4 = w1 + 2*w2; subplot(3, 1, 1), stem(nx, w1), set(get(gca, 'Title'), 'String', 'w_1[n]'); subplot(3, 1, 2), stem(nx, w2), set(get(gca, 'Title'), 'String', 'w_2[n]'); subplot(3, 1, 3) stem(nx, w3, 'm--s') hold on stem(nx, w4, 'r--p') set(get(gca, 'Title'), 'String', 'w_3[n] & w_4[n]'); legend('w_3[n]', 'w_4[n]', 'Location', 'southeast'); saveas(gcf, "P2_5_a_3_out.png"); 2.5(d) a = [1 -3/5]; b = 1; x = [1 zeros(1,19)]; h1 = filter(b, a, x); h2 = zeros(1,21); nh = 0:19; for i = 2:21 h2(i) = ((3/5)^(i-2)) * h2(i-1) + x(i-1) end h2 = h2(2:end); figure; subplot(2, 1, 1); stem(nh, h1); title('y_1[n]=(3/5)y_1[n-1]+x[n]'); xlabel('n'); ylabel('h_1[n]'); subplot(2, 1, 2); stem(nh, h2); title('y_2[n]=(3/5)^ny_1[n-1]+x[n]'); xlabel('n'); ylabel('h_2[n]'); saveas(gcf, "P2_5_d_out.png"); 2.5(e) u = ones(1, 20); s1 = filter(b, a, u); s2 = zeros(1,21); ns = 0:19; for i = 2:21 s2(i) = ((3/5)^(i-2)) * s2(i-1) + u(i-1) end s2 = s2(2:end); figure; subplot(2, 1, 1); stem(ns, s1); title('y_1[n]=(3/5)y_1[n-1]+x[n]'); xlabel('n'); ylabel('s_1[n]'); subplot(2, 1, 2); stem(ns, s2); title('y_2[n]=(3/5)^ny_1[n-1]+x[n]'); xlabel('n'); ylabel('s_2[n]'); saveas(gcf, "P2_5_e_out.png"); 2.5(f) z1 = conv(h1, u); z1 = z1(1:20); z2 = conv(h2, u); z2 = z2(1:20); nz = 0:19; figure; subplot(2, 1, 1); stem(nz,z1); title('z1[n]'); xlabel('n'); ylabel('z1[n]'); subplot(2, 1, 2); stem(nz, z2); title('z2[n]'); xlabel('n'); ylabel('z2[n]'); saveas(gcf, "P2_5_f_out.png"); 2.5(g) figure; subplot(3, 1, 1); hold on; stem(nz, s1, 'm--s'); stem(nz, z1, 'r--p'); legend('s_1[n]', 'z_1[n]', 'Location', 'northeast'); title('s_1[n] & z_1[n]'); xlabel('n'); ylabel('s_1[n] / z_1[n]'); subplot(3, 1, 2); stem(ns, s2); title('s_2[n]'); subplot(3, 1, 3); stem(nz, z2); title('z_2[n]'); saveas(gcf, "P2_5_g_out.png"); 2.10 % Problem 2.10(a) load lineup.mat %sound(y, 8192); figure; plot(y) title('Sound befor filtering'); saveas(gcf, "plots/P2_10_ini_out.png"); close; % 2.10(a) x = [1 zeros(1, 1000)]; nx = 0:1000; he = zeros(1, 1001); nhe = 0:1000; for i = 1:1001 if i < 1001 he(i) = x(i); else he(i) = x(i) + 0.5*x(i-1000); end end figure; stem(nhe, he); title('h_e[n]'); xlabel('n'); saveas(gcf, "plots/P2_10_a.png"); close; % 2.10(b) % This is not a problem that require to be verified by MATLAB A = [1 zeros(1, 999) 0.5]; B = [1]; z = filter(B, A, y); figure; stem(conv(z,he),'*'); hold on; stem(y,'p--g') title('y[n] & z[n]*he[n]'); xlabel('n'); saveas(gcf, "plots/P2_10_b.png"); close; % 2.10(c) a = [1 zeros(1, 999) 0.5]; b = [1]; d = [1, zeros(1, 4000)]; nd = 0:4000; her = filter(b, a, d); nher = 0:4000; figure; stem(nd, her); title('h_{er}[n]'); xlabel('n'); saveas(gcf, "plots/P2_10_c.png"); close; % 2.10(d) % I feel that the given value 1000 and 0.5 are fine. a = [1 zeros(1, 999) 0.5] z = filter(1, a, y); figure; plot(z); title('after filtering sound'); saveas(gcf, "plots/P2_10_d.png"); %close; sound(z, 8192); % 2.10(e) hoa = conv(her, he); nhoa = (nher(1)+nhe(1)):(nher(end)+nhe(end)); figure; stem(nhoa, hoa); title('h_{oa}[n]'); xlabel('n'); saveas(gcf, "plots/P2_10_e.png"); close; % Problem 2.10(f) load lineup.mat A = [1 zeros(1, 999) 0.5]; B = [1]; z = filter(B, A, y); z=z'; rxx=conv(z, fliplr(z)); % For y1 ryy1 = conv(y, flip(y)); nryy1 = -(length(y)-1):(length(y)-1); figure; plot(nryy1, ryy1); title('y[n]*y[-n]'); saveas(gcf, "plots/P2_10_f1_1.png"); close; a1=rxx(7000);b1=2*rxx(8000);c1=rxx(7000)-ryy1(7000); d1=b1^2-4*a1*c1; x1=(-b1+d1^0.5)/(2*a1); N = 1000; yo1 = filter(1, [1 zeros(1, N-1) x1], y); sound(yo1, 8192); % For y2 ryy2 = conv(y2, flip(y2)); nryy2 = -(length(y2)-1):(length(y2)-1); figure; plot(nryy2, ryy2); title('y_2[n]*y_2[-n]'); saveas(gcf, "plots/P2_10_f1_2.png"); close; a2=rxx(7000);b2=2*rxx(7501);c2=rxx(7000)-ryy2(7000); d2=b2^2-4*a2*c2; x2=(-b2+d2^0.5)/(2*a2); N = 501; yo2 = filter(1, [1 zeros(1, N-1) x2], y2); sound(yo2, 8192); % For y3 ryy3 = conv(y3, flip(y3)); nryy3 = -(length(y3)-1):(length(y3)-1); figure; plot(nryy3, ryy3); title('y_3[n]*y_3[-n]'); saveas(gcf, "plots/P2_10_f1_3.png"); close; Note: Please indicate meaning of the symbols in all expressions. Please indicate the coordinate and unit in all figures. |  |
| Experience You can write your experience with this project. Any comment and suggestion on this course are also very welcome. |  |
| Score |  |

字体：英文Times new Roman；中文宋体，正文五号

文件名统一命名方式：LabX+姓名+学号，例如：Lab1+张三+00001 （正式报告删除此行！）