Lab 4 The Continuous-Time Fourier Transform

| Author | Name： 赵奕帆 崔俞崧 Student ID:11812020 11811305 |
| --- | --- |
| Introduction In the lab, we learned the Continuous Time Fourier Transform. And we have three problems in the lab. In 4.2, we learned how to compute the numerical approximation of CTFT. In 4.5, we will learn how to find analytic expressions for the impulse responses of stable LTI systems whose inputs and outputs satisfy linear constant-coefficient differential equations. In 4.6, we will explore amplitude modulation of Morse code messages. Lab results & Analysis： 4.2 Here, xt=gt+g-t, and gt=e-2tut. So, Xjw= 12+jw+12-jw. tau=0.01; T=10; N=T/tau; t=(0:tau:T-tau); y=exp(-2 * abs(t-5)); Y = fftshift(tau * fft(y)); Here, we can use the fftshift to get the value of Y(jw). w = -(pi/tau) + (0:N-1)*(2*pi / (N*tau)); (The output is too large to put the screenshot here.) X = exp(li*5*w).*Y; We can get that Xjw= ej5wY(jw). We can find that the difference of approximate X(jw) is small to theoretical X(jw) in low frequency, and is bigger in high frequency. We can find that the abs magnitude of X(jw) and Y(jw) are same, but the angles are the different. The reason is that y(t) is time shifting of x(t). 4.5 We would find that the Hjω= Y(jω)X(jω)= jω-2jω2+1.5jω+0.5, so a1=1 1.5 0.5, b1=1-2. We find that p1=-1-0.5, r1=6-5, then we get Hjω= Y(jω)X(jω)= jω-2jω2+1.5jω+0.5= 6jω+1+ -5jω+0.5. From the problem (a) and problem (b), we know that the h1t=6e-tut-5e-0.5tu(t) so it’s integrable. Hjω= Y(jω)X(jω)= 3jω2+10jω+5jω3+ 7(jω)2+16jω+12, then we find a2=1 7 16 12, b2=3 10 5. We find that p2=-3-2-2 , r2=2 1-3, then we get Hjω= Y(jω)X(jω)= jω-2jω2+1.5jω+0.5= 2jω+3 + 1jω+2 - 3(jω+2)2. From the problem (d) and problem (e), we know that the h2t=2e-3tut+e-2tut-3te-2tu(t) so it’s integrable. Hjω= Y(jω)X(jω)= -4(jω)2-4, then we find a3=1 0-4, b3=-4. We find that p2=-1 1, r2=2-2, then we get Hjω= Y(jω)X(jω)= -4(jω)2-4= 1jω+2- 1jω-2. From the problem (d) and problem (e), we know that the h3t=e-2tut-e2tut, because the h3t≠0 t<0, so it’s not causal. 4.6 Z = [dash dash dot dot]; In this case, we can see that most of the energy in the Fourier transformation will move beyond the filter's passband. y1t ↔ 14M[jω-4πf1+2jω+j(ω+4πf1)] y2t ↔ 14jM[jω-4πf1-j(ω+4πf1)] y3t ↔ 14M[jω-2πf1-2πf2+jω+2πf1-2πf2+jω-2πf1+2πf2+jω+2πf1+2πf2 We may find that m1=dash dot dot, so the letter is D. We may find that m2=dot dot dot, so the letter is S. We may find that m2=dash dash dot, so the letter is P. So it’s DSP in altogether. 代码： 代码： 4.2 % 4.2(a) % 4.2(b) tau=0.01; T=10; N=T/tau; t=(0:tau:T-tau); y=exp(-2 * abs(t-5)); %4.2(c) Y = fftshift(tau*fft(y)); %4.2(d w = -(pi/tau)+(0:N-1)*(2*pi/(N*tau)); %4.2(e) X = exp(1i*5*w).*Y; %4.2(f) magnitudeX_a = abs(X); phaseX_a = angle(X); X2 = 1 ./ (2 + 1j * w) + 1 ./ (2 - 1j * w); magnitudeX = abs(X2); phaseX = angle(X2); figure; subplot(2, 1, 1); semilogy(w, magnitudeX_a , w, magnitudeX); legend('approximation ¦X¦', '¦X¦ ', 'Location', 'northeast'); title('X magnitude and X approximation'); subplot(2, 1, 2); hold on; semilogy(w, phaseX_a, w, phaseX); legend('approximation ¦X¦', '¦X¦', 'Location', 'northeast'); title('X phase and X approximation'); saveas(gcf, "P4_2_out1.png") close; %4.2(g) magnitudeY = abs(Y); phaseY = angle(Y); figure; subplot(2, 1, 1); semilogy(w, magnitudeY , 'r--h',w, magnitudeX_app, 'b'); legend('Y', 'X', 'Location', 'northeast'); title('magnitude of X and Y'); subplot(2, 1, 2); hold on; semilogy(w, phaseY, 'b'); semilogy(w, phaseX, 'g'); legend('Y', 'X', 'Location', 'northeast'); title('phase of X and Y'); saveas(gcf, "P4_2_out2.png") 4.5 % 4.5_a a1 = [1 1.5 0.5]; b1 = [1 -2]; % 4.5_b [r1, p1] = residue(b1, a1); % 4.5_d a2 = [1 7 16 12]; b2 = [3 10 5]; [r2, p2] = residue(b2, a2); % 4.5_g a3 = [1 0 -4]; b3 = -4; % 4.5(h) [r3, p3] = residue(b3, a3); 4.6 load ctftmod.mat % 4.6_a z = [dash dash dot dot]; % 4.6_b figure; freqs(bf, af) title('Frequency response of bf and af'); saveas(gcf, "./P4_6_b.png") close % 4.6_c ydash = lsim(bf, af, dash, t(1:length(dash))); ydot = lsim(bf, af, dot, t(1:length(dot))); figure; subplot(2,1,1) hold on; plot(t(1:length(dash)), dash); plot(t(1:length(dash)), ydash); legend('dash without lowpass filter', 'dash with lowpass filter', 'Location', 'northeast'); title('Dash'); xlabel('t'); subplot(2,1,2) hold on; plot(t(1:length(dot)), dot); plot(t(1:length(dot)), ydot); legend('dot without lowpass filter', 'dot with lowpass filter', 'Location', 'northeast'); title('Dot'); xlabel('t'); saveas(gcf, './P4_6_c.png') close % 4.6_d y = dash .* cos(2*pi*f1*t(1:length(dash))); yo = lsim(bf, af, y, t(1:length(y))); figure; subplot(2,1,1) hold on; plot(t(1:length(dash)), y); title('Without filter'); subplot(2,1,2) plot(t(1:length(y)), yo); title('With filter'); saveas(gcf, './P4_6_d.png') close % 4.6_g x1 = x .* cos(2*pi*f1*t(1:length(x))); m1 = lsim(bf, af, x1, t(1:length(x1))); x2 = x .* sin(2*pi*f2*t(1:length(x))); m2 = lsim(bf, af, x2, t(1:length(x2))); x3 = x .* sin(2*pi*f1*t(1:length(x))); m3 = lsim(bf, af, x3, t(1:length(x3))); figure; subplot(3,1,1) plot(t, m1); title('m1'); xlabel('t'); subplot(3,1,2) plot(t, m2); title('m2'); xlabel('t'); subplot(3,1,3) plot(t, m3); title('m3'); xlabel('t'); saveas(gcf, "./P4_6_g.png") close Note: Please indicate meaning of the symbols in all expressions. Please indicate the coordinate and unit in all figures. |  |
| Experience You can write your experience with this project. Any comment and suggestion on this course are also very welcome. |  |
| Score |  |