Lab 1：Introduction

| Author | Name：袁通/代育津 Student ID: 11810818/11910114 |
| --- | --- |
| Introduction In this LAB session, the most important task for us is to learn how to use matlab to analysis basic signals, we will use matlab to explore the property of the functions and write a different function in a Discrete-time System. The many requirements of this lab are listed here: - Learn to construct a input signal to test the property of the system. - Learn the analysis the property of a system. Lab results & Analysis： Problem 1.4 Discrete-time systems are often characterized in terms of a number of properties such as linearity, time invariance, stability, causality, and invertibility. It is important to understand how to demonstrate when a system does or does not satisfy a given property. MATLAB can be used to construct counter-examples demonstrating that certain properties are not satisfied. In this exercise, you will obtain practice using MATLAB to construct such counterexamples fcr a variety of systems and properties. Basic Problems For these problems, you are told which property a given system does not satisfy, and the input sequence or sequences that demonstrate clearly how the system violates the property. For each system, define MATLAB vectors representing the input(s) and output(s). Then, make plots of these signals, and construct a well reasoned argument explaining how these figures demonstrate that the system fails to satisfy the property in question. Task (a): Solution: As shown in the figure, in Figure 1.4(a) x1n=δ[n] is applied to system y1n=sin(π2x1[n]) and x2n=2δ[n] is applied to system y2n=sin(π2x2[n]). Here we have x2n=2x1[n] but we can’t obtain y2n=2y1[n] so the system is not linear. Task (b): Solution: Fig 1.4(b)-1 demonstrate the signal xn=u[n], Fig 1.4(b)-2 demonstrate the signal xn+1=u[n+1], Fig 1.4(b)-3 demonstrate the signal output of system yn=xn+x[n+1]. Because in Fig 1.4(b)-3 y[-1]≠0, so the system is not causal. Intermediate Problems For these problems, you will be given a system and a property that the system does not satisfy, but must discover for yourself an input or pair of input signals to base your argument upon. Again, create MATLAB vectors to represent the inputs and outputs of the system and generate appropriate plots with these vectors. Use your plots to make a clear and concise argument about why the system does not satisfy the specified property. Task(c): Solution: For the given interval (0,2), the range of the function is (-∞,log2), so the system is not stable. Task(d): Solution: Fig 1.4(d)-1 is the signal x1n=δn, Fig 1.4(d)-2 is the signal x2n=5δn, and in Fig 1.4(d)-3 and Fig 1.4(d)-4 we apply x1[n] and x2[n] to the system yn=sin(π2x[n]). We found that though x1n≠x2n but y1n=y2[n] so different input obtain the same output, the system is not invertible. For each of the following systems, state whether or not the system is linear, time-invariant, causal, stable, and invertible. For each property you claim the system does not possess, construct a counter-argument using MATLAB to demonstrate how the system violates the property in question. Advanced Problems For each of the following systems, state whether or not the system is linear, time-invariant, causal, stable, and invertible. For each property you claim the system does not possess, construct a counter-argument using MATLAB to demonstrate how the system violates the property in question. Task(e): yn=x3[n] Solution: The system does not satisfy linearity. In the figure we apply x1[n]=u[n] and x2n=3u[n] to the system yn=x3n. We found that though x2n=3x1[n], we cannot obtain y2n=y1n here so the system is not linear. Task(f): yn=nx[n] Solution: Stability, time-invariant and invertibility not satisfied. Figure 1.4(f)-1 indicates the output of the system y1[n]=nxn with x1[n]=n, Figure 1.4(f)-2 indicates the output of the system y2[n]=nxn with x2[n]=δ[n], Figure 1.4(f)-3 indicates the output of the system y3[n]=nxn with x3[n]=3x2[n], and in Figure 1.4(f)-4, we apply y4'n=y2n+1=n+1x[n+1], y4n=y4'n-1=n+1xn . We found that y[n] will grow without bound so the system is not stable, though x3[n]=3x2[n] but we obtain y3n=3y2n so the system is not invertible, also, we found that y4[n]≠y2[n] so the system does not satisfy time-invariant. Task(g): yn=x[2n] Solution: The system is not causal, time-invariant, stability. Firstly, Figure 1.4(g1)-1 indicates x1n=u[n+2], Figure 1.4(g1)-2 indicates y1n=x12n=u[n+1], then Figure 1.4(g1)-3 indicates x2n=x1n+2=un+4, Figure 1.4(g1)-4 indicates y2n=x22n=u[n+2], and Figure 1.4(g1)-5 indicates y1n+2=u[n+3]≠y2 so the system is not time invariant. Secondly, we found that if xn=δn-2 as shown in Figure 1.4(g2)-1 and yn=x2n=δn-1 which means the future x is used as input, so the system is not causal. Thirdly, Figure 1.4(g3)-1 indicate the signal x1n=δ[n+1], Figure 1.4(g3)-2 indicate the signal y1n=x12n=δ[n+12], Figure 1.4(g3)-3 indicate the signal x2n=δ[n+3], Figure 1.4(g3)-1 indicate the signal y2n=x2[2n]=δ[n+32], we can found that with different input signal x1 and x2 , we will obtain same output, so the system is not invertible. Problem 1.5 Task(a) Task(b) Solution: In Figure 1.5(b)-1, we apply x1[n]=δ[n] to the differential function and we get a stable output with y1n=1, and in Figure 1.5(b)-2, we apply x2[n]=u[n] to the differential function and we get the output with y2n=n. Task(c): Solution: Figure 1.5(c)-1 indicate the output y1[n] with input x1n=u[n], Figure 1.5(c)-2 indicate the output y2[n] with input x2n=2u[n], and Figure 1.5(c)-3 indicates the result of difference 2×y1n-y2n . We can found that the difference is always equal to -1, not identically equal to 0, because that here we have y-1=-1 in the differential function, so we have y10=0 and y20=1 , obviously the system is not linear. Task(d): Solution: Figure 1.5(d)-1 indicates the y1[n] with y1-1=0, xn=u[n] and a=0.5, we can found that the y[n] begins at 1 and gradually close to 2, and Figure 1.5(d)-2 indicates the y2[n] with y2-1=0.5, xn=u[n] and a=0.5, we can found that the y[n] begins at 1.25 and gradually close to 2. We found that for yn=a×y-1+x0 n=0a×yn-1+xn n≥1 We have y0=a×y-1+x[0] y1=a×y0+x1=a2×y-1+a×x0+x[1] yn=an+1×y-1+i=1nan×x[i-1]+x[n] So for y1[n] and y2[n] the only difference at each item y[n] is an+1×y-1 which will decrease as n increase while ¦a¦<1 Appendix: MATLAB Code Problem 1.4: Task a: n = -5:1:5; x1=[zeros(1,5) 1 zeros(1,5)]; x2=2*x1; y1=sin((pi/2)*x1); y2=sin((pi/2)*x2); %Plot stem(n,y1,'g--*'); hold on; stem(n,y2,'r--s'); legend('y_1=sin((\pi/2) x_1[n]','y_2=sin((\pi/2) x_2[n]') title('1.4 (a)'); xlabel('n'); ylabel('y[n])'); saveas(gcf, "LAB1/P1_4_a_out.png") Task b: n1=-5:9; n2=-6:9; x1=[zeros(1,5),ones(1,10)]; x2=[zeros(1,4),ones(1,11)]; subplot(3,1,1); stem(n1,x1,'b--^'); xlim([-6 10]); title('1.4(b)-1'); xlabel('n'); ylabel('x[n]=u[n]'); subplot(3,1,2); stem(n1,x2,'b--^'); xlim([-6 10]); title('1.4(b)-1'); xlabel('n'); ylabel('x[n+1]=u[n+1]'); y1=[0 x1+x2]; subplot(3,1,3); stem(n2,y1,'r--p'); xlim([-6 10]); title('1.4(b)-2'); xlabel('n'); ylabel('y[n]=x[n]+x[n+1]'); saveas(gcf, "LAB1/P1_4_b_out.png") Task c: x=0:0.03:2; y=log(x); stem(x,y,'m--h'); title('1.4(c)'); xlabel('x'); ylabel('y[n]=log(x[n])'); saveas(gcf, "LAB1/P1_4_c_out.png") Task d n= -5:1:5; x1=[zeros(1,5) 1 zeros(1,5)]; x2=5*[zeros(1,5) 1 zeros(1,5)]; y1=sin((pi/2)*x1); y2=sin((pi/2)*x2); subplot(3,1,1); stem(n,x1); title('1.4 (d)-1'); xlabel('n'); ylabel('x_1[n]'); subplot(3,1,2); stem(n,x2); title('1.4 (d)-2'); xlabel('n'); ylabel('x_2[n]'); subplot(3,1,3); stem(n,y1,'b--s'); hold on; stem(n,y2,'r--h'); legend('(sin(\pi/2) x_1[n])','(sin(\pi/2) x_2[n])'); title('1.4 (d)-3'); xlabel('n'); ylabel('y[n]'); saveas(gcf, "LAB1/P1_4_d_out.png") Task e: % Linearity not satified n=-5:1:5; x1=n>=0; x2=3*x1; y1=x1.^3; y2=x2.^3; stem(n,y1); hold on stem(n,y2,'r'); title('1.4(e)'); legend({'x[n]=u[n]','x[n]=3u[n]'}, 'Location', 'northeast'); xlabel('x[n]'); ylabel('y[n]=x^3[n]'); saveas(gcf, "LAB1/P1_4_e_out1.png") Task f % Stability analysis n=-5:1:5; x1=n; y1=n.*x1; subplot(4,1,1) stem(n,y1,'g--s'); title('1.4(f)-1'); xlabel('x_1[n]=n'); ylabel('y_1[n]=nx_1[n]'); % invertibility and time-invariant analysis x2=[zeros(1,5) 1 zeros(1,5)]; x3=3 .* x2 y2=n .* x2; y3=n .* x3; y4=(n+1) .* x2; subplot(4,1,2); stem(n,y2,'r--h'); title('1.4(f)-2'); xlabel('x_2[n]'); ylabel('y_2[n]'); subplot(4,1,3); stem(n,y3,'r--h'); title('1.4(f)-3'); xlabel('x_3[n]'); ylabel('y_3[n]'); subplot(4,1,4); stem(n,y4,'m--^'); title('1.4(f)-4'); xlabel('x_2[n]'); ylabel('y_4[n]'); saveas(gcf, "LAB1/P1_4_f_out2.png") Task g %不满足时不变性、因果性、可逆性 %时不变性分析,由3，4图的对比可知. % Time invariant n=-5:1:5; x1=n>=-2; y1=n>=-1; x2=n>=-4; y2=n>=-2; y3=n>=-3; subplot(3,2,1); stem(n,x1); title('1.4(g1)-1'); xlabel('n') ylabel('x_1[n]=u[n+2]'); subplot(3,2,2); stem(n,y1); title('1.4(g1)-2'); xlabel('n') ylabel('y_1[n]=u[n+1]'); subplot(3,2,3); stem(n,x2); title('1.4(g1)-3'); xlabel('n') ylabel('x_2[n]=x_1[n+2]=u[n+4]'); subplot(3,2,4); stem(n,y2); title('1.4(g1)-4'); xlabel('n') ylabel('y_2[n]=x_2[2n]=u[n+2]'); subplot(3,2,5); stem(n,y3); title('1.4(g1)-5'); xlabel('n') ylabel('y_1[n+2]'); saveas(gcf, "LAB1/P1_4_g1_out2.png") %因果性分析,x[n]超前了 % Causal n=-5:1:5; x1=[zeros(1,7) 1 zeros(1,3)]; y1=[zeros(1,6) 1 zeros(1,4)]; subplot(2,1,1); stem(n,x1); title('1,4(g2)-1'); xlabel('x[n]=δ[n-2]') subplot(2,1,2); stem(n,y1); title('1.4(g2)-2'); xlabel('y[n]=x[2n]=δ[n-1]') saveas(gcf, "LAB1/P1_4_g2_out.png") % Invertable analysis n=-5:1:5; x1=[zeros(1,4) 1 zeros(1,6)] y1=[zeros(1,11)] x2=[zeros(1,2) 1 zeros(1,8)] y2=[zeros(1,11)] subplot(2,2,1); stem(n,x1); title('1.4(g3)-1'); xlabel('x_1[n]=δ[n+1]'); subplot(2,2,2); stem(n,y1); title('1.4(g3)-2'); xlabel('y_1[n]'); subplot(2,2,3); stem(n,x2); title('1.4(g3)-3'); xlabel('x_2[n]=δ[n+3]'); subplot(2,2,4); stem(n,y2); title('1.4(g3)-4'); xlabel('y_2[n]'); saveas(gcf, "LAB1/P1_4_g3_out2.png") Problem 1.5: Task a function y=diffeqn(a,x,yn1) y=zeros(length(x)); y(1)=a*yn1+x(1); if length(x)>=2 for i=2:length(x) y(i)=a*y(i-1)+x(i); end end end Task b: n=0:1:30; x1=n==0; x2=n>=0; a=1; yn1=0; y1=diffeqn(a,x1,yn1); y2=diffeqn(a,x2,yn1); subplot(2,1,1); stem(n,y1,'r--p'); title('1.5(b)-1') xlabel('x1[n]=δ[n]'); subplot(2,1,2); stem(n,y2,'m--s'); title('1.5(b)-2') xlabel('x2[n]=u[n]'); saveas(gcf, "LAB1/P1_5_b_out.png") Task c: n=0:1:30; x1=n>=0; x2=2.*x1; a=1; yn1=-1; y1=diffeqn(a,x1,yn1); y2=diffeqn(a,x2,yn1); y3=2.*y1-y2; subplot(3,1,1); stem(n,y1,'r--p'); title('1.5(c)-1'); xlabel('x_1[n]=u[n]'); subplot(3,1,2); stem(n,y2,'m--s'); title('1.5(c)-1'); xlabel('x_2[n]=2*u[n]'); subplot(3,1,3); stem(n,y3,'^'); title('1.5(c)-1'); xlabel('2*y_1[n]-y_2[n]'); saveas(gcf, "LAB1/P1_5_c_out.png") Task d: n=0:1:30; x=n>=0; a=0.5; yn1=0; y1=diffeqn(a,x,yn1); yn1=0.5; y2=diffeqn(a,x,yn1); stem(n,y1,'r--p'); legend('y[-1]=0'); hold on; stem(n,y2,'b--s'); legend('y[-1]=0.5'); title('1.5(d)'); saveas(gcf, "P1_5_d_out.png") Note: Please indicate meaning of the symbols in all expressions. Please indicate the coordinate and unit in all figures. |  |
| Experience In this LAB session we firstly established the workflow of the computational research for signal and system with matlab, which ensure our efficiency of research, secondly we learnt more operations of matlab which we did not use in the past, and finally we learnt how to analysis the property of a system seriously. Also, at first we have some difficulty in analysis some of the properties like causality though we draw the right plots to prove it, but after discussion we finally get it. More details could be found here. |  |
| Score |  |

字体：英文Times new Roman；中文宋体，正文五号